Anh Nguyen

017623292

10/24/2019

CECS 361 Fall 2019

Project 2: VGA Synchronization Design

Requirements Testing

Reset brings the VGA Synchronization circuit to a known state with all outputs inactive

The Synchronization Logic shall be updated at the 25Mhz

The Horizontal Scan Count shall be updated at the 25MHz

The Horizontal Scan Count shall range from 0 to 799

The Horizontal Sync signal shall be LOW ACTIVE and shall be active from Horizontal Scan Count 656-751

The Horizontal Video On signal shall be HIGH ACTIVE and shall be active from Horizontal Scan Count 0-639

The Vertical Scan Count shall be updated at the completion of a Horizontal Scan

The Vertical Scan Count shall range from 0-524

The Vertical Sync signal shall be LOW ACTIVE and shall be active from Vertical Scan Count 490-491

The Vertical Video On signal shall be HIGH ACTIVE and shall be active from Vertical Scan Count 0-479

The Video On signal shall be HIGH ACTIVE and shall be active when Horizontal Video On and Vertical Video On are active at the same time

The RGB signals shall be driven while the Video On signal is ACTIVE. When the Video On signal is INACTIVE the RGB signals shall be held at 0